| HAZID |  |  |  |
| --- | --- | --- | --- |
| Biodiesel Manufacturing - Methanol Tank Storage |  |  |  |
| HAZ ID# | Hazard | Cause | Consequences |
| 1 | Thermal Expansion - Methanol - Vapor generation | Faulty/blocked vent - Excessive In-tank vapor generation | Over-pressurization - Loss of containment. Property & tank damage |
| 2 | Corrosion - Methanol tanks, trim & piping | Incompatible tank and trim materials | Loss of containment. Methanol product loss. |
| 3 | Human factors/errors - Methanol tanks - overfilling | Distraction, miscalculation | Methanol product loss. |
| 4 | Environmental conditions - carbon steel tanks -corrosion | Proximity to ocean and salts | Methanol product loss. |
| 5 | Natural disasters - methanol tanks/piping - earthquake | Storage system not designed for seismic activity | Methanol product loss. |
| 6 | Ignition sources - bonding & grounding - static discharge | inadequate conductivity in bonding & grounding | Damage to equipment from fire and explosion |